ARINC 717 Flight Data Transmission System

Run Guide — Commands, Outputs & How to Verify Everything Works

# System Overview

There are 5 Python files. Each can be run standalone to test its own stage, and pipeline.py ties them all together. All files must be in the same folder.

| File | What It Does |
| --- | --- |
| arinc_generator.py | Generates 88 flight parameters as 12-bit ARINC 717 words (1200 sec, 300 frames) |
| compressor.py | Delta-encodes + zstd compresses telemetry + 60-sec CVR audio. ~4.4x ratio, ~3 min Tx |
| packet_framer.py | Splits bundle into 1200B chunks, signs each with Ed25519, builds manifest |
| quic_transport.py | Sends/receives chunks over libp2p QUIC. Per-chunk ACK, RTT, retransmit metrics |
| pipeline.py | Orchestrator: runs all stages in sequence. Entry point for airplane and ground modes |

# Step 0 — Install Dependencies

Run once before anything else:

pip install zstandard lz4 libp2p pynacl numpy

libp2p install takes 1-2 min and installs aioquic, trio, nacl automatically.

# Step 1 — arinc_generator.py

## Command

python arinc_generator.py

## What it does

Generates 20 minutes (1200 seconds) of synthetic flight data for 88 parameters. Each parameter has a realistic sample rate (1/sec to 8/sec) and waveform (sinusoidal altitude, noisy vibration, stepwise flaps, etc.). Packs everything into 12-bit ARINC 717 words with proper sync words.

## Actual output you will see

================================================================

[GEN] ARINC 717 Telemetry — 88 params, 1200s (300 frames)

================================================================

[SLOT MAP] Slots used per SF: [97, 37, 59, 37]  (max 255)

Parameters       : 88

Total subframes  : 1200  (300 frames)

Total words      : 307,200

Raw bytes        : 460,800

Raw Tx @ 7 kbps  : 8.78 min

First 5 slot assignments:

Altitude_ft      rate=4/s  slots=[(0, 1), (1, 1)]...

Airspeed_kts     rate=4/s  slots=[(0, 2), (1, 2)]...

Pitch_deg        rate=8/s  slots=[(0, 3), (0, 4)]...

Roll_deg         rate=8/s  slots=[(0, 5), (0, 6)]...

Yaw_deg          rate=4/s  slots=[(0, 7), (1, 7)]...

## How to verify it worked

Good signs to look for:

• 'Parameters: 88' — all 88 parameters generated

• 'Slots used per SF: [97, 37, 59, 37]' — each number must be < 255 (max slots)

• 'Total words: 307,200' — exactly 256 words/sec × 1200 sec

• 'Raw bytes: 460,800' — exactly 307,200 × 1.5 (12-bit packing = 3 bytes per 2 words)

• 'Raw Tx @ 7 kbps: 8.78 min' — uncompressed data would need 8.78 min, inside 10-min limit

If you see 'assert 88' error: one of the PARAM_DEFS entries was deleted. File must have exactly 88 param tuples.

# Step 2 — compressor.py

## Command

python compressor.py

## What it does

Compresses the 88-param telemetry + a 60-second CVR audio snippet. Uses delta encoding first (reduces entropy since telemetry changes slowly), then zstandard at max level. Also packs into the .a717 bundle binary format with a SHA256 integrity header.

## Actual output you will see

[GEN] ...  (generator output first)

Telem: 460,800B  CVR(60s): 240,000B

Compressed: 158,365B  CR:4.43x  Tx:3.02min

Within 10min budget: True

Round-trip: telem=OK  cvr=OK

## How to verify it worked

• 'CR:4.43x' — compression ratio, expect 3.5x to 6x on this data

• 'Tx:3.02min' — transmission time at 7kbps, must be less than 10.00

• 'Within 10min budget: True' — the critical check

• 'Round-trip: telem=OK  cvr=OK' — decompress → must match original exactly

If Round-trip shows FAIL: delta_decode has a bug. Check numpy cumsum overflow.

CVR note: Full 20-min audio at 8kHz = 150+ min over 7kbps — physically impossible. We send 60-sec snippet at 4kHz 8-bit only.

# Step 3 — packet_framer.py

## Command

python packet_framer.py

## What it does

Splits the compressed bundle into 1200-byte chunks. Signs each chunk individually with Ed25519 (64-byte signature). Builds a manifest chunk (sent first) that lists total chunks, expected CRCs, and the public key. The receiver uses the manifest to detect missing chunks without waiting for a timeout.

## Actual output you will see

=== Packet Framer Test ===

Bundle size: 134,631 bytes

[SIGNER] Public key: e4eab90a6c437d5a8643...

[FRAMER] Chunking 134,631 bytes into 113 chunks of 1200B

[FRAMER]   bundle_id=33885397b5c34d77...

[FRAMER] Chunk header overhead: 102B per chunk

[FRAMER] Manifest chunk: 1,778B

[FRAMER] Total wire bytes: 147,935  overhead: 9.9%

Manifest: n_chunks=113  sha256=9140174ead89d383...

Reassemble: {'total_expected': 113, 'total_received': 113,

'chunks_ok': 113, 'chunks_bad_crc': 0,

'chunks_bad_sig': 0, 'missing_seqs': []}

Bytes match: True

## How to verify it worked

• 'chunks_ok: 113' — every chunk passed CRC and Ed25519 signature check

• 'chunks_bad_crc: 0' — no data corruption during chunking/parsing

• 'chunks_bad_sig: 0' — all signatures verified with the correct key

• 'missing_seqs: []' — no chunks lost in the round-trip

• 'Bytes match: True' — reassembled bytes are bit-for-bit identical to original bundle

If chunks_bad_sig > 0: key mismatch. Verifier must use the same public key as Signer.

The public key hex printed here is what you pass to the ground station via --pubkey flag.

# Step 4 — quic_transport.py

## Receiver command (run on ground station first)

python quic_transport.py receiver 8000

This starts the QUIC receiver on port 8000. It will print the full peer multiaddress you need for the sender:

[RECV] libp2p QUIC receiver started

[RECV] Peer ID: 16Uiu2HAm...

[RECV] Listening: /ip4/192.168.1.10/udp/8000/quic-v1/p2p/16Uiu2HAm...

## Sender command (run on airplane side, paste address from above)

python quic_transport.py sender \

/ip4/192.168.1.10/udp/8000/quic-v1/p2p/16Uiu2HAm...

## What sender prints during transfer

[SENDER] Started  port=8001

[SENDER] Connecting (attempt 1)...

[SENDER] Connected to 16Uiu2HAm...

[SENDER] Stream open  protocol=/arinc717/1.0.0

[SENDER] Sending manifest (1778B)...

[SENDER] Manifest acknowledged

[SENDER] chunk   1/113    0.9%  RTT=12.3ms

[SENDER] chunk  50/113   44.2%  RTT=8.7ms

[SENDER] chunk  67/113   59.3%  RTT=9.1ms  RETX=1   <-- retransmit example

[SENDER] chunk 113/113  100.0%  RTT=10.2ms

[SENDER] Final status: 'RECV_COMPLETE'

============================================================

TRANSPORT METRICS  session=a3f2c1b0

============================================================

Flight ID         : FLT0001

Elapsed           : 162.4 s

Bytes sent        : 147,935

Throughput        : 7,293 bps

Chunks sent       : 113

Chunks acked      : 113

Chunks lost       : 0

Loss rate         : 0.00%

Retransmissions   : 1

RTT avg           : 10.4 ms

RTT min           : 7.2 ms

RTT max           : 31.6 ms

RTT p99           : 28.9 ms

Retransmit chunks : [66]

============================================================

## What receiver prints

[RECV] Incoming stream  protocol=/arinc717/1.0.0

[RECV] Manifest: 113 chunks expected  bundle=33885397...

[RECV] 50/113   44.2%  missing=63

[RECV] 113/113  100.0%  missing=0

[RECV] All chunks received (113). Reassembling...

[RECV] SHA256 verified ✓

[RECV] Saved bundle: ./received_bundles/33885397_1700000000.a717

## How to verify it worked

• Sender prints 'RECV_COMPLETE' as final status — receiver confirmed everything

• 'SHA256 verified ✓' on receiver side — data was not corrupted in transit

• Loss rate 0.00% on a local network is normal; over real HF expect 1-5%

• 'missing=0' at 100% — no chunks lost

• File appears in ./received_bundles/ folder

If you see 'RECV_PARTIAL': some chunks were lost and not retransmitted. Increase CONNECT_RETRY in quic_transport.py.

If connection refused: make sure receiver is running BEFORE sender. They must be on the same network or routable IP.

# Step 5 — pipeline.py (Full System)

## Self-test (no network needed — run this first)

python pipeline.py test

Runs the entire pipeline on one machine without any network. Generates data, compresses, chunks, signs, reassembles, decompresses, and verifies bit-for-bit equality. Takes about 10-15 seconds.

Look for this at the end:

Telemetry round-trip: PASS ✓

CVR round-trip:       PASS ✓

Tx @ 7 kbps    : 2.82 min

SELF TEST COMPLETE

## Ground station (run this first, on the receiving machine)

python pipeline.py ground --port 8000 --flight-id FLT0001

Starts the QUIC receiver and waits. Prints the full bootstrap address like:

/ip4/192.168.1.10/udp/8000/quic-v1/p2p/16Uiu2HAm9HRU...

Copy that entire address. You need it for the airplane side.

## Airplane side (run after ground is ready)

python pipeline.py airplane \

--peer /ip4/192.168.1.10/udp/8000/quic-v1/p2p/16Uiu2HAm... \

--distress-at 10 \

--flight-id FLT0001 \

--sender-port 8001

--distress-at 10 means: simulate a distress event trigger 10 seconds after start (for testing). Remove this flag in production and trigger manually.

## Output files created by airplane side

| File | Contents |
| --- | --- |
| pipeline_output/FLT0001_bundle.a717 | The compressed+signed binary bundle (local backup) |
| pipeline_output/FLT0001_pubkey.hex | Ed25519 public key hex — give this to ground station |
| pipeline_output/FLT0001_tx_metrics.json | Full transmission metrics: RTT, loss, throughput, chunk log |

## Output files created by ground side

| File | Contents |
| --- | --- |
| received_bundles/<id>_<ts>.a717 | The received bundle (same as what airplane sent) |
| pipeline_output/FLT0001_telemetry.bin | Decompressed raw 12-bit telemetry binary |
| pipeline_output/FLT0001_cvr.pcm | Decompressed 60-sec CVR audio (4kHz 8-bit raw PCM) |

## Optional: use airplane's public key on ground for signature verification

# First get the key printed by airplane side, then:

python pipeline.py ground \

--port 8000 \

--pubkey <hex string from pipeline_output/FLT0001_pubkey.hex>

If pubkey is not provided, signature verification is disabled with a WARNING. Always use a key in production.

# Understanding the Metrics Output

## Transport metrics — what each number means

| Metric | What it tells you |
| --- | --- |
| Chunks sent / acked | Total chunks dispatched vs acknowledged. Should match unless link failed completely |
| Chunks lost | Chunks that were never ACKed after all retransmit attempts. 0 = perfect delivery |
| Loss rate % | chunks_lost / chunks_sent × 100. On a local network expect 0%. Over HF expect 1-5% |
| Retransmissions | Total times a chunk was resent. Higher than 0 means some packets were lost then recovered |
| RTT avg / min / max | Round-trip time per chunk in milliseconds. Includes network + receiver processing |
| RTT p99 | 99th percentile RTT. High p99 vs avg = occasional bad delays (link fade, congestion) |
| Throughput bps | Actual bytes transferred / elapsed time × 8. Should be close to 7000 bps on HF link |
| Retransmit chunks | List of which chunk sequence numbers were retransmitted — useful for debugging link quality |
| Lost seqs | List of permanently lost chunk seq numbers. Reassembler will flag these as missing |

## Reassembly report — what each field means

| Field | What it tells you |
| --- | --- |
| total_expected | From manifest: how many chunks the sender said it would send |
| total_received | How many chunks actually arrived |
| chunks_ok | Chunks that passed both CRC32 and Ed25519 signature. Should equal total_received |
| chunks_bad_crc | Data was corrupted in transit (bit flip). Should be 0 — QUIC itself does checksumming |
| chunks_bad_sig | Signature mismatch — chunk came from wrong sender, or key is wrong. Should be 0 |
| missing_seqs | List of sequence numbers never received. These are the chunks that need retransmit |

# Troubleshooting

| Error / Symptom | Fix |
| --- | --- |
| 'dns.resolver.NoResolverConfiguration' | Add nameserver: echo 'nameserver 8.8.8.8' >> /etc/resolv.conf |
| 'ModuleNotFoundError: libp2p' | Run: pip install libp2p --break-system-packages |
| 'ModuleNotFoundError: zstandard' | Run: pip install zstandard |
| Cannot connect to peer | Start receiver (ground) BEFORE sender (airplane). Both must be on same network. Check firewall allows UDP on port 8000 |
| 'protocol not supported' error | Handler must be registered before host.run(). This is already fixed in quic_transport.py |
| Round-trip FAIL in test | Delta decode has a bug. Should not happen with current code — check numpy version (need ≥1.20) |
| Tx time > 10 min | CVR duration is too long. In compressor.py change simulate_cvr_audio(60) — 60 is the max for budget |
| 'chunks_bad_sig > 0' | Key mismatch between Signer and Verifier. Make sure ground loads the pubkey saved by airplane side |
| Sender hangs at 'Connecting' | Receiver is not running, or firewall blocking UDP. Try: nc -u <ip> 8000 |

# Data Flow (Text Diagram)

AIRPLANE SIDE                          GROUND SIDE

─────────────────                      ─────────────────

arinc_generator.py                     quic_transport.py

88 params, 300 frames    ─────────►    receive chunks

12-bit words + sync                    verify CRC+sig

│                                 reassemble

compressor.py                          decompress

delta + zstd                           save .a717 file

4.43x ratio                                 │

3.02 min Tx                         pipeline_output/

│                                telemetry.bin

packet_framer.py                      cvr.pcm

Ed25519 sign each chunk

1200B chunks + manifest

│

quic_transport.py

send over QUIC

per-chunk ACK + RTT

metrics JSON

# Quick Reference Card

| What you want to do | Command |
| --- | --- |
| Test everything without a network | python pipeline.py test |
| Test only telemetry generation | python arinc_generator.py |
| Test only compression | python compressor.py |
| Test only chunk signing | python packet_framer.py |
| Start ground station receiver | python pipeline.py ground --port 8000 |
| Start airplane sender | python pipeline.py airplane --peer <addr> --distress-at 10 |
| Start raw QUIC receiver (no full pipeline) | python quic_transport.py receiver 8000 |
| Start raw QUIC sender (no full pipeline) | python quic_transport.py sender <addr> |

Generated by ARINC 717 Transmission System